TOWN OF MENOMONIE

TOWN ROAD STANDARDS FOR NEW ROAD CONSTRUCTION

The Town Board of the Town of Menomonie, Dunn County, Wisconsin, ordains as follows:

I.     Town Road Standards: Subdivision Roads - subject to Subdivision Ordinance,

Residential Roads, or other roads to meet the following standards before being

accepted by the Town of Menomonie.

A. Town Road Standards shall meet minimum geometric design  pursuant to

Wisconsin Statutes  86.26 and  86.265. Minimum standards shall not be less

than 86.26 (c).

1.    Right of Way         4 rods (66 feet)

2.    Roadway Width     26 feet

3.    Surface Width       22 feet

4.    Maximum Grade    8 to 11%

B. Development or other roads shall not be in violation of the Dunn County

Comprehensive Zoning Ordinance.

C. Minimum Standards maybe increased by action of the Town Board which

includes Right-of-way, Roadway Width, Surface Width, Grade and Curvature.

D. An engineering / plat survey showing elevation, grade, curvature, etc. shall be

submitted and approved by the board prior to acceptance of the road plan by the

Town Board. Prior to acceptance and approval of the completed roadway, this

engineering / plat shall be signed off by the engineer stating that the roadway was

completed to these standards.

E. Other Standards:

Drainage / Ditching:   Storm, culverts and related facilities shall be designed to permit

the unimpeded flow of natural water courses, insure the drainage of all low points

along the line of the roadway. provide positive drainage away from on-site sewer

disposal facilities; and, facilitate infiltration of on-site runoff where possible. Three (3)

to Five (5) feet to the ditch bottom. Fillslopes / backslopes 2:1 maximum.

When it become necessary to make a lateral trench leading away form the main

ditch; then the additional land necessary for removal of the accumulated water, must

be provided and deeded over to the Town of Menomonie, along with the necessary

land to the highway. This land conveyed to the town for drainage, will be under the

supervision of the Town of Menomonie at all times.

Sub Base Course:  Must be of a quality and composition suitable for the location. In

low or swampy areas this course must have a sandy composition, to provide

necessary drainage of the road bed. Any muck holes encountered before and during

construction of the road bed, must be removed and filled with a sandy lift to provide

a solid base. Sub grade shall be inspected and approved by the engineer and town-

ship prior to placing base course and top soil in the ditches.

Erosion Control:   Top soil and seed ditches / establish grass cover. Signed off by the

engineer stating all requirements completed as dictated by present law.

Base Course:   Must consist of crushed packing gravel of a quality and composition,

suitable for traffic loads. The amount of gravel necessary for acceptance must be at

least Six (6) inches for a base of crushed limestone or Seven (7) inches base of

Wisconsin #2 gravel, measured after being compacted.

Asphalt:   Must include a minimum of 2 inch covering of the surface which will consist

of the blacktop portion - minimum blacktop portion will be 22 feet wide.

Culverts:   The minimum length of any culvert installed in the road bed shall be 30 feet

in length. Minimum culvert size shall be at least 18 “ with a cover of at least 12” to the

top of the sub base.

If at any time it is decided by the Town Board, that the construction of a culvert or

bridge would be of a size and cost that it would create a hardship to the owner or

owners of the land required to build said culvert of bridge; then the board will proceed

to accept the road, complete as required by this ordinance except the part extending

100 feet on each side of the culvert or bridge known as the approach. This approach

will be accepted incomplete. The town will then proceed to build the said culvert or

bridge and approach with the help of bridge aid, if available, the balance of the cost of

construction not covered by aid, will be charged to the owner or owners of the abutting

road.

Cul-de-sac:   The closed end shall have a turn - around with a radius of Forty Three

Feet (43)  surfaced, and a right of way radius if Fifty Five Feet (55).

Guardrails / Reflective Signage:   Shall be placed on curves or steep embankments to

insure the safe travel of vehicular traffic. The board will determine the need for

guard rail, signage, and approve the placement and type.

II.     Completion:    Upon completion of the road, the board of supervisors will proceed to

make final inspection, accepting or rejecting the road as the case may be. If the road

is rejected, then corrections must be made as recommended by the board, before

final inspection can then be made again. If final acceptance is then made, the owner or

owners will turn it over to the town, the deed of all land necessary for the road. This

road then shall be adopted and submitted for yearly state highway aids.

III.     Effective:   This ordinance shall repeal any previous board action on standards

for new road construction and take effect from and after its passage and publication

as provided by law.

Adopted this ______ day of ___________,. 2001 by the Town Board of the Town of

Menomonie, Dunn County, Wisconsin.

Chairman     ________________________

Supervisor    ________________________

Supervisor    _______________________

Filed this ____ day of ______________, 2001.

Published ________________________ Town Clerk   ________________

TOWN OF MENOMONIE

TOWN ROAD STANDARDS FOR NEW ROAD CONSTRUCTION

ENGINEER / CONTRACTOR / OWNER SIGN OFF RECORD

Section  1  D    Plat / survey

Engineer _____________________________

Township _____________________________

Date        _____________________________

Section  1  E    Sub Base Course

Engineer _____________________________

Township ____________________________

Date        _____________________________

Section  1  Erosion Control

Engineer _____________________________

Date        _____________________________

Section  11  Completion

Engineer _____________________________

Owner     _____________________________

Township _____________________________

Date         _____________________________

Three copies       Engineer      Owner      Town of Menomonie